Circuits Bazaar
Contact: +91-8684-8844-88

# AUTOMATIC THREE PHASE CHANGER USING RELAYS

A Detailed Project Synopsis

Submitted for partial fulfillment of academic requirements
in Electrical / Electronics Engineering

# 1. Introduction

Electrical power distribution systems commonly face issues related to phase failure, voltage imbalance, and intermittent power availability. In many residential, commercial, and industrial locations, a three-phase supply is available, but the connected loads are often single-phase in nature. When one phase fails, the load experiences a complete power outage despite the presence of other healthy phases.

The Automatic Three Phase Changer project is designed to overcome this limitation by automatically shifting the connected load to the next available healthy phase without manual intervention. This system ensures uninterrupted power supply and enhances reliability, safety, and convenience. The project is implemented purely using electrical and electronic components such as relays, comparators, rectifiers, and control logic, without the use of any microcontroller, making it robust and easy to maintain.

# 2. Problem Statement

In conventional power systems, phase failure is one of the most common faults. Statistics show that a majority of power interruptions occur due to single-phase faults while the remaining phases remain operational. Manual phase changeover is unsafe, time-consuming, and impractical, especially during odd hours or in unmanned installations.

There is a strong need for an automatic system that can continuously monitor all phases, identify phase failure conditions, and instantly transfer the load to a healthy phase without causing electrical hazards or supply interruptions.

# 3. Objectives of the Project

The main objectives of this project are:
- To ensure uninterrupted single-phase power supply from a three-phase source.
- To automatically detect phase presence and absence.
- To eliminate manual phase selection.
- To provide visual and audible indication of phase status.
- To design a reliable system without using any microcontroller.
- To improve electrical safety and operational efficiency.

# 4. System Overview

The Automatic Three Phase Changer continuously monitors the availability of three input phases. Each phase is independently sensed using step-down transformers and rectifier circuits. The sensed signals are processed by relay driver circuits that control high-power relays. The relays are interconnected in such a way that only one phase supplies the load at any given time.

In case the currently selected phase fails, the system instantly switches the load to the next available phase in a predefined priority sequence.

# 5. Components Used and Detailed Description

## 5.1 Step-Down Transformers

Step-down transformers are used to reduce the high AC voltage (230V AC) from each phase to a safer low-voltage level (typically 12V AC). These low-voltage signals are used for phase detection and control circuitry. The transformers also provide electrical isolation between the high-voltage power lines and the low-voltage control circuit.

## 5.2 Rectifier and Filter Circuit

Each transformer output is connected to a rectifier circuit, usually consisting of diodes, to convert AC voltage into pulsating DC. Capacitors are then used to filter out ripples, providing a relatively smooth DC voltage suitable for relay driver and indicator circuits.

## 5.3 Voltage Regulator (LM7805)

The LM7805 voltage regulator is used to provide a stable 5V DC output for low-power control components such as comparators, LEDs, and buzzers. Voltage regulation ensures reliable operation despite fluctuations in input voltage.

## 5.4 Relays

Electromagnetic relays form the core switching mechanism of the system. High-power relays are used to connect the load to the selected phase. The relays ensure electrical isolation between control and power circuits and allow safe handling of high currents.

## 5.5 Relay Driver Circuit

Relay driver circuits are built using transistors such as BC547 along with base resistors and flyback diodes. These circuits amplify low-current control signals to levels sufficient to energize relay coils while protecting components from back EMF.

## 5.6 Indicators and Buzzer

LED indicators are used to visually display the presence or absence of each phase. A buzzer provides an audible alert during phase failure conditions, enhancing user awareness and safety.

## 5.7 Wiring, Connectors, and Protection Devices

High-quality wires, terminal connectors, fuses, and DPDT switches are used to ensure safe and reliable operation. Proper insulation and grounding are critical to prevent electrical hazards.

# 6. Circuit Diagram

# 7. Block Diagram

# 8. Working Principle

Under normal operating conditions, all three phases are continuously monitored. When the preferred phase is available, its corresponding relay remains energized, supplying power to the load. If that phase fails, the control circuit immediately de-energizes the active relay and energizes the next relay connected to a healthy phase.

This automatic switching happens within milliseconds, ensuring minimal disruption to the load.

# 9. Advantages

- Ensures uninterrupted power supply.
- No manual intervention required.
- Simple and robust design.
- No dependency on microcontrollers or software.
- Low maintenance and high reliability.

# 10. Disadvantages

- Limited flexibility compared to programmable systems.
- Mechanical wear of relays over long-term use.
- Not suitable for extremely high-power industrial loads without contactors.

# 11. Applications

The Automatic Three Phase Changer is widely applicable in:
- Residential buildings.
- Small and medium industries.
- Commercial complexes.
- Hospitals and clinics.
- Rural and semi-urban power systems.

# 12. Future Scope

Future enhancements may include integration with smart monitoring systems, replacement of relays with solid-state devices, incorporation of remote monitoring, and expansion to handle higher power ratings using contactors.

# 13. Conclusion

The Automatic Three Phase Changer project successfully demonstrates a reliable solution for maintaining uninterrupted power supply using conventional electrical components. Its simplicity, safety, and effectiveness make it suitable for both educational and practical applications.

Circuits Bazaar | Contact: +91-8684-8844-88